# Laptop Repair-and-Resell Guideline

ShopSphere Retail — TechNova Ultrabook 14 and other laptops

| Guideline ID | RFB-LT-2025-01 |
| --- | --- |
| Version | 1.4 |
| Effective date | 2025-06-01 |
| Category | electronics (laptops) |
| Recovery methods | repair_and_resell, refurbish_and_resell, supplier_return |

## 1. When repair-and-resell applies

A returned laptop qualifies for repair_and_resell when the inspection shows a single repairable fault, all other diagnostics pass, the serial matches the order line, and the repair cost is below 35% of the expected refurbished resale value.

## 2. Keyboard damage

Cracked or missing keycaps, broken scissor mechanisms and non-registering keys are repaired by replacing the keyboard/top-case assembly. Individual keycap replacement is not used on TechNova Ultrabook 14 because of clip fragility.

Check for liquid indicators before repair. If any are triggered, route to refurbish_and_resell with a full board inspection.

Typical bench time: 2.5 hours, plus a 4-hour burn-in and a full key-matrix test.

## 3. Worked example

Return R108 (order O108, serial LT2044): inspection IR-2026-00008 recorded keyboard damage (J and K keycaps cracked, right Shift missing, J and right Shift not registering) with all other diagnostics passing and battery health at 94%. This meets the criteria for repair_and_resell: replace the keyboard/top-case, burn in, regrade, and list as Refurbished Grade A if the cosmetic check passes.

## 4. Grading after repair

| Grade | Cosmetic | Battery health |
| --- | --- | --- |
| Refurbished A | No visible marks at 30 cm | >= 85% |
| Refurbished B | Light marks on lid or base | >= 75% |
| Refurbished C | Visible wear; fully functional | >= 65% |

## 5. Data security

All storage is wiped to NIST 800-88 Purge level before any repair-and-resell or refurbishment. The wipe certificate is attached to the recovery task.